DOE Grant Proposal — Kill Brine Research Findings

Supplement to DOE_Minerals_KillBrine_Technical_Reference.docx

Energy Water Solutions  |  Research compiled August 4, 2026  |  Sources cited inline

# 1. Completion Fluids Market Size & Context

## 1.1 Global Market

- Global drilling and completion fluids market: $10.3B (2025), growing at 4.2% CAGR to $12.9B by 2032. Source: Allied Market Research, Future Market Insights, Zion Market Research (multiple corroborating reports).

- North America = ~55% of global market (~$5.7B). Driven by Permian Basin completions and Gulf of America deepwater.

- Water-based fluids = ~60% of market, oil-based = 30%, synthetic = 10%.

- Completion fluids specifically (clear brines, not drilling mud) is a subset — dominated by TETRA Technologies, Albemarle, ICL Group, Newpark.

## 1.2 TETRA Technologies — Public Benchmark

TETRA (NYSE: TTI) is the largest pure-play completion fluids company and the only oilfield service provider that manufactures its own calcium chloride and heavy brines. Their financials provide the best public benchmark for the kill brine market:

| Metric | Value | Source |
| --- | --- | --- |
| Completion Fluids & Products Q4 2024 quarterly revenue | $69M (27.3% EBITDA margin) | TETRA Q4 2024 Earnings Release (Feb 25, 2025) |
| Completion Fluids annualized run-rate (Q4 2024) | ~$276M/year | Extrapolated from $69M quarterly |
| Total TETRA 2024 revenue | ~$540M (all segments) | TETRA FY2024 results |
| Total TETRA 2025 revenue guidance | $620–630M (raised twice) | TETRA Q3 2025 Earnings (Oct 28, 2025) |
| Completion Fluids 2025 YoY revenue growth | +50% YoY (2025 vs 2024) | EarningsIQ TETRA analysis (Aug 2026) |
| Completion Fluids 2025 EBITDA margin | 33% (up 420 bps YoY) | EarningsIQ TETRA analysis (Aug 2026) |
| TETRA 2030 target | $1.2–1.3B total revenue | TETRA Investor Day / Gurufocus analysis |
| Arkansas bromine plant | Phase 1 completed below budget; 40% above supply agreement output | TETRA Q4 2025 Earnings (Feb 25, 2026) |

## 1.3 Permian Kill Fluid Market Estimate

Bottom line: Ian's 30–40,000 bpd figure is likely in the right order of magnitude for the Permian-specific kill-weight completion brine market, but the total addressable market is larger.

- Each Permian horizontal completion uses 300,000–500,000 bbl of total water (frac fluid + produced water for sourcing). Of this, completion/workover fluid is a smaller, denser subset.

- ~450 active frac crews in the Permian (2024–2025). At ~2 wells/month/crew, that's ~900 completions/month.

- If each completion uses ~5,000–15,000 bbl of heavy brine (10+ ppg), total demand = 4.5–13.5M bbl/month = 150,000–450,000 bpd.

- The kill-weight brine subset (CaCl₂, CaBr₂, ZnBr₂ — 10.0+ ppg) is narrower: ~50,000–150,000 bpd.

- At $20–80/bbl for CaCl₂ brine (low end) and $100–300/bbl for CaBr₂/ZnBr₂ (high end), the Permian kill brine market is $400M–$1.5B/year.

- EWS's 10-unit deployment would produce ~2,850 bpd of brine — roughly 2–6% of the Permian kill brine market. Niche, not saturated.

## 1.4 Key Industry Players

| Company | Role | Relevance to EWS |
| --- | --- | --- |
| TETRA Technologies (NYSE: TTI) | Largest pure-play completion fluids provider. Manufactures CaCl₂, CaBr₂, ZnBr₂ brines. Only OFS company that makes their own CaCl₂. | Potential partner or competitor. They buy CaCl₂ feedstock — EWS brine could be a supply source. HQ in The Woodlands, TX (45 min from Tomball). Brady Murphy, CEO. |
| Albemarle (NYSE: ALB) | WELLBROM® clear brine fluids. Sodium/calcium bromide based. Also the world's largest lithium producer. | Bromine supplier, not competitor. Their WELLBROM products are bromide-based completion fluids. Potential bromine off-take partner from EWS mineral extraction. |
| ICL Group (NYSE: ICL) | World-leading bromine-based clear brine fluids manufacturer. Bromine from Dead Sea. | Bromine chemistry expertise. Potential mineral extraction partner or technology collaborator for iodine/bromine separation. |
| Newpark Resources (NYSE: NR) | Full line of clear brine fluids, filtration units, and additives. Completion fluids + drilling fluids. | Smaller than TETRA in completion fluids. Also HQ in The Woodlands. Potential conditioning partner — they sell filtration units. |
| Select Water Solutions (NYSE: WTTR) | Water management + completions. Produced water sourcing for fracs. | Competitor in produced water reuse. Their business model validates the produced-water- to-completion-fluid concept. |

# 2. Verified Commercial Specifications

## 2.1 ISO 13503-3:2022 — The Governing Standard

- Title: "Petroleum and natural gas industries — Completion fluids and materials — Part 3: Testing of heavy brines"

- Covers: physical properties, potential contaminants, and test procedures for heavy brine fluids manufactured for oil and gas well drilling, completion, and workover fluids.

- Replaces ISO 13503-3:2005. Updated 2022.

- Paired with API RP 13J (5th Edition, 2014) — "Testing of Heavy Brines" — the operational field standard.

## 2.2 Key Performance Parameters

From the Drilling Fluids Explored technical reference (corroborated by ISO 13503-3 and API RP 13J):

| Parameter | Specification | Test Method | EWS Relevance |
| --- | --- | --- | --- |
| Density (ppg / SG) | 8.4–19.2 ppg Tolerance: ±0.1 ppg | Mud balance (API RP 13J §5) | VOX output at 70% evap: 9.9 ppg median. Can target higher via more evap or CaCl₂ addition. ±0.1 tolerance is achievable. |
| Clarity / Turbidity | <25–50 NTU (solids-free) | Turbidimeter (API RP 13J §7) | CRITICAL GAP: VOX output will contain precipitated BaSO₄, SrSO₄, CaSO₄, possibly NaCl. Filtration REQUIRED. |
| Crystallization Temperature | Must remain liquid at BHST and surface (typically <−20°C) | ISO 13503-3 §8 Fann CT tester | CaCl₂ brines crystallize −20 to −50°C. Not a Permian issue (ambient >0°C). CaBr₂/ZnBr₂ brines go much lower. |
| pH | Typically 6–9 | pH meter (API RP 13J §6) | GAP: VOX evaporation generates HCl via MgCl₂/CaCl₂ hydrolysis. pH may drop to 4–5. Needs caustic/lime addition. |
| Total Suspended Solids (TSS) | <50–100 mg/L | Gravimetric / filtration test | Precipitated salts from concentration. Bag/cartridge filtration + settling. |
| Formation Compatibility | No swelling, scaling, or permeability damage | Core flood testing (API RP 13J §10) | Year 1 activity at university partner. CaCl₂ brines generally compatible with sandstone and carbonate formations. |
| Iron Content | Varies by formation (typically <50 mg/L) | ICP / titration (ISO 13503-3 §9) | Permian median Fe = 17 mg/L feed. At 10× = 170 mg/L in brine. Chelation or oxidation/filtration may be needed. |

## 2.3 Brine Types and Density Ranges

The standard clear brine fluid types, ordered by density (ISO 13503-3 / API RP 13J):

| Brine Type | Density Range (ppg) | Primary Salt | EWS Production Viability |
| --- | --- | --- | --- |
| Potassium chloride (KCl) | 8.4–9.0 | KCl | Not relevant — too dilute. EWS brine starts well above this range. |
| Sodium chloride (NaCl) | 8.4–10.0 | NaCl | EWS brine at 50% evap achieves this. Lowest value tier. Baseline case. |
| Sodium bromide (NaBr) | 8.4–12.5 | NaBr | Requires bromine from mineral extraction. Mid-tier value. 0.5% of Permian wells. |
| Calcium chloride (CaCl₂) | 8.5–11.6 | CaCl₂ | EWS SWEET SPOT. 62.8% of Permian wells at 70% evap. No imported salt needed. |
| Calcium bromide (CaBr₂) | 11.0–14.2 | CaBr₂ | Requires bromine extraction. Premium pricing. 0.05% of Permian wells naturally. Must formulate with extracted Br. |
| Calcium chloride/ bromide blends | 11.6–14.2 | CaCl₂ + CaBr₂ | Mixed salt. EWS could formulate from concentrated CaCl₂ brine + extracted bromine. |
| Zinc bromide (ZnBr₂) | 14.2–19.2 | CaBr₂ + ZnBr₂ | Highest density. Zinc is toxic — TETRA's zinc-free fluids are a differentiator. EWS should NOT pursue zinc. |
| Cesium formate | Up to 19.2+ | CsCOOH | Niche HPHT. Not relevant to Permian. |

# 3. Conditioning Process — From VOX Output to Spec Brine

## 3.1 Current VOX Output State

- Temperature: 80–90°C (operating temp at tower base).

- Density: 9.9 ppg median (at 70% evap of median Permian feed). Can go higher with more evaporation.

- Solids: Precipitated CaSO₄ (gypsum), BaSO₄ (barite), SrSO₄ (celestite), possibly NaCl (halite) at >85% evap.

- pH: Likely 4–6 due to HCl generation from MgCl₂/CaCl₂ hydrolysis (not neutral as feed water).

- Iron: ~56 mg/L (from 17 mg/L feed × 3.3× concentration).

## 3.2 Required Unit Operations (in order)

These are standard oilfield unit operations. No novel technology required. System can be skid-mounted.

| Step | Operation | Equipment | Purpose | Est. Cost |
| --- | --- | --- | --- | --- |
| 1 | Cooling | Shell-and-tube or plate heat exchanger | Bring brine from 80–90°C to ambient (<40°C) for safe handling and next steps | $30–80K |
| 2 | Settling / Coarse Filtration | Settling tank + bag filter (50–100 µm) | Remove bulk precipitated solids (gypsum, barite, celestite) | $20–50K (tank + filters) |
| 3 | Fine Filtration | Cartridge filter (5–25 µm nominal) | Achieve <50 NTU clarity per API RP 13J | $15–30K (vessel + elements) |
| 4 | pH Adjustment | Chemical injection: caustic soda (NaOH) or hydrated lime | Neutralize HCl from evaporation, raise pH to 6–9 | $10–20K (tank + pump) |
| 5 | Density Adjustment | Dilution water or additional evaporation | Hit target ppg ±0.1. Dilute if over-concentrated; evaporate more if under-concentrated. | Included in VOX op cost |
| 6 | Iron Removal (if needed) | Aeration + filtration or chelation (EDTA) | Remove dissolved iron for sensitive formations. May be optional. | $15–25K |
| 7 | Biocide Treatment | Chemical injection: glutaraldehyde or THPS | Prevent bacterial growth if brine stored before use | $5–10K |
| 8 | Quality Testing | Mud balance, turbidimeter, pH meter, filtration test | Verify spec compliance per ISO 13503-3/API RP 13J. Per-batch QA/QC. | $10–15K (instruments) |

Total estimated conditioning skid cost: $150–250K per pad (1,000 bpd brine output). Range is wide because iron removal and fine filtration are site-specific. Get vendor quotes before locking the budget number.

## 3.3 Integration with Mineral Extraction

The conditioning step follows mineral extraction in the process flow. This is important for the proposal narrative:

- 1. VOX evaporation → concentrated brine (~285 g/L TDS)

- 2. Slipstream (100–200 bpd) → Neighbors/Hexagon media → iodine + bromine extraction

- 3. Main brine stream → conditioning skid (cool, filter, pH adjust, density trim)

- 4. Spec-grade kill brine → tank storage → truck to customer location

- Critical unknown: Does iodine/bromine extraction change brine chemistry in ways that affect kill brine grade? Neighbors claims halogens are selectively removed (shouldn't strip Ca), but this needs verification with actual post-extraction brine samples.

# 4. Potential Partners & Consultant Targets

## 4.1 Completion Fluids Companies (Partner or Customer)

| Company | HQ | Approach |
| --- | --- | --- |
| TETRA Technologies | The Woodlands, TX (45 min from Tomball) | Largest and most relevant. They manufacture their own CaCl₂ — EWS brine could be a feedstock. Brady Murphy, CEO. Approach: 'We're building a DOE-funded brine concentration platform. Your Arkansas bromine operation needs CaCl₂ feedstock — we can supply it from produced water at lower cost.' |
| Newpark Resources | The Woodlands, TX | Smaller, more likely to partner. They sell filtration units for completion fluids — exactly the equipment EWS needs. Approach: 'We're piloting produced-water-to-kill-fluid. Your filtration units are the conditioning step. Partnership gets your equipment into a DOE-funded demonstration.' |
| Albemarle | Charlotte, NC | If bromine extraction is the priority. WELLBROM products use their bromine — EWS could be a domestic bromine source. Approach: 'You import bromine from the Dead Sea and Chile. We can produce it domestically from Permian produced water.' |
| Select Water Solutions | Houston, TX | They already source produced water for fracs. EWS adds the concentration/mineral extraction layer. Potential: 'You source the water, we concentrate it, we both sell the brine.' |

## 4.2 Individual Consultants / Domain Experts

Target profile: Completions engineer with 15+ years experience in completion fluid design, preferably with operating company or service company background. Needs to be willing to put resume/letter in the DOE proposal.

- Path 1: Steep Shiner (EWS internal — inventor of the box, former completions engineer). Does he have contacts from his completions days?

- Path 2: SPE Completions community. SPE Gulf Coast Section has a Completions Study Group. Reach out to chair for consultant referrals.

- Path 3: Retired TETRA/Newpark/Baker Hughes completions engineers. These companies have been through multiple M&A cycles — plenty of experienced people available for consulting.

- Path 4: University of Texas Petroleum Engineering department — faculty with completion fluid research background. Dr. Mukul Sharma, Dr. Eric van Oort.

- Path 5: LinkedIn search for "completion fluids consultant" or "brine specialist" in Houston/Austin/Midland. Dozens of independents.

## 4.3 What the Letter/Resume Needs to Say

For the DOE proposal, the consultant/partner letter should address:

- 1. Statement of qualifications in completion fluid design/formulation.

- 2. Acknowledgement of the EWS brine concentration approach and its viability for kill fluid production.

- 3. Commitment to provide technical oversight for brine conditioning to meet ISO 13503-3 / API RP 13J specs.

- 4. Description of testing protocol to validate brine quality (core flood, compatibility, density, clarity).

- 5. Role in techno-economic analysis: validating the market price and demand for the brine product.

# 5. Permian Water Context — Why This Matters

- Permian Basin produces 6+ million bopd (2024) and 20–30 million bpd of produced water (3–5× oil ratio; up to 10:1 in Delaware Basin).

- Disposal capacity is tightening — injection constraints, seismicity regulations, and regulatory scrutiny on SWD wells.

- Produced water reuse for completions is growing: operators now source 20–40% of frac water from produced water rather than freshwater.

- EWS's value proposition to DOE: turn a disposal liability (produced water) into two revenue-generating products (minerals + kill brine) while reducing SWD injection volumes.

- The 'beneficial reuse' narrative is exactly what DOE's water management funding priorities target.

Energy Water Solutions — CONFIDENTIAL — Kill Brine Research Findings  |  August 2026

Sources: TETRA Technologies SEC filings & earnings releases, ISO 13503-3:2022, API RP 13J, Drilling Fluids Explored, Allied Market Research, SPE JPT