DOE Grant Proposal — Technical Reference

Mineral Extraction Economics + Kill Brine Feasibility

Energy Water Solutions  |  August 2026  |  USGS PWDB v2.3

# 1. Mineral Extraction Economics

Data source: USGS Produced Waters Geochemical Database v2.3 — 15,792 Permian Basin records, 344–434 wells with iodine/bromine/lithium data. All values are feed-water concentrations (pre-evaporation). Evaporation at 70% multiplies concentrations ~3.3× and revenue proportionally.

## 1.1 Ranking: Formations by Total Mineral Revenue (¢/bbl feed)

| Formation | Basin | Iodine mg/L | Iodine ¢/bbl | Bromine ¢/bbl | Lithium ¢/bbl | Total ¢/bbl | TDS g/L | Kill Brine Grade | Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spraberry/Strawn/Fusselman | Midland | 193 | 153.4 | 23.4 | — | 176.8 | 114 | CaCl₂-grade | Iodine-First ⭐ |
| Cline Shale | Midland | 160 | 127.2 | 42.5 | — | 169.7 | 41 | None (too dilute) | Iodine-First |
| Spraberry Lower | Midland | 139 | 110.7 | 44.4 | 4.2 | 159.3 | 160 | CaCl₂-grade | Iodine-First |
| Atoka | Midland | 110 | 87.1 | 13.4 | 0.8 | 101.3 | 69 | Low-end CaCl₂ | Iodine-First |
| Spraberry | Midland | 71 | 56.3 | 32.1 | 2.3 | 90.7 | 144 | CaCl₂-grade | Balanced ⭐ |
| Wolfcamp Shale | Midland | 65 | 51.5 | 26.4 | — | 77.9 | 116 | Marginal | Iodine-First |
| Cherry Canyon | Delaware | 21 | 16.6 | 35.7 | 4.0 | 56.3 | 230 | Premium CaCl₂ | Brine-First |
| Dean | Midland | 46 | 36.2 | 17.7 | 2.8 | 56.7 | 142 | Low-end CaCl₂ | Balanced |
| Bell Canyon | Delaware | 18 | 14.3 | 28.0 | 2.0 | 44.3 | 234 | Premium CaCl₂ | Brine-First |
| Wolfcamp | Midland | 39 | 31.3 | 17.8 | 2.7 | 51.8 | 95 | Marginal | Iodine-First |
| Queen | Delaware | 3 | 2.5 | 10.3 | 1.6 | 14.4 | 96 | Low-end CaCl₂ | Brine-First |
| Yates | Delaware | 2 | 1.7 | 7.7 | 1.2 | 10.6 | 73 | NaCl kill brine | Brine-First |

## 1.2 Basin-Level Comparison

| Metric | Midland Basin (3,612 wells) | Delaware Basin (1,290 wells) |
| --- | --- | --- |
| Iodine (median) | 17.0 mg/L → 13.5¢/bbl | 10.5 mg/L → 8.3¢/bbl |
| Bromine (median) | 240.6 mg/L → 11.5¢/bbl | 306.0 mg/L → 14.6¢/bbl |
| Lithium (median) | 11.7 mg/L → 2.2¢/bbl | 8.5 mg/L → 1.6¢/bbl |
| TDS (median) | 110,888 mg/L | 63,610 mg/L |
| Ca for kill brine | 4,331 mg/L → 43 g/L brine | 1,508 mg/L → 15 g/L brine |
| Best mineral formation | Spraberry (90.7¢/bbl total) | Cherry Canyon (56.3¢/bbl) |
| Best brine formation | Cherry Canyon (230 g/L TDS) | Bell Canyon (234 g/L TDS) |
| Dominant strategy | Iodine + CaCl₂ brine | Kill brine first, minerals secondary |

## 1.3 Revenue at Scale (Spraberry, 10 Units × 1,000 bpd)

Spraberry P90 scenario (288 mg/L iodine — top decile Spraberry wells):

| Revenue Stream | ¢/bbl feed | Annual Revenue (10K bpd) | % of Total | Notes |
| --- | --- | --- | --- | --- |
| Iodine (I) | 56.3¢ | $2.05M/yr | 55% | Median Spraberry; P90 = 229¢ → $8.34M |
| Bromine (Br) | 32.1¢ | $1.17M/yr | 31% | Used in oilfield fluids — captive market |
| Kill Brine (CaCl₂) | ~40¢ | $1.46M/yr | 13% | After I+Br extraction, brine still CaCl₂-grade |
| Lithium (Li) | 2.3¢ | $83K/yr | 1% | Rounding error — not commercially viable alone |
| TOTAL | 130.7¢ | $4.77M/yr | 100% | P90 iodine = 229¢ → $10.9M/yr total |

## 1.4 The I-TDS Ceiling Effect

Fundamental geological constraint: you cannot maximize iodine AND kill brine from the same formation.

| TDS Range | Iodine Range | Examples | Strategy |
| --- | --- | --- | --- |
| >200 g/L | 0–22 mg/L | Cherry Canyon, Bell Canyon, Penn Detrital | Kill brine = 70–85% of revenue |
| 100–150 g/L | 23–105 mg/L | Spraberry, Wolfcamp, Dean | Iodine = 48–75% of revenue |
| <60 g/L | Up to 160 mg/L | Cline Shale | Iodine only — zero brine revenue |

Geological cause: iodine comes from organic-rich source rocks (kerogen). High-TDS brines are ancient evaporated seawater that never contacted source rocks. Correlation is −0.10 (weak linear) but the ceiling is absolute: zero wells above 200 g/L TDS exceed 22 mg/L iodine.

## 1.5 Formation Classification Matrix

| Type | Formations | Revenue Split | Best Use Case |
| --- | --- | --- | --- |
| Type 1: Brine-First | Cherry Canyon, Bell Canyon, McKee, Odom | Brine 70–85% Iodine 0–8% | Customer cares about completion fluid. Premium brine grade. Low mineral upside. |
| Type 2: Iodine-First | Cline Shale, Atoka, Wolfcamp Shale | Iodine 48–75% Brine 0–25% | Customer wants mineral revenue. Cline has best iodine but zero brine. Atoka is borderline. |
| Type 3: Balanced ⭐ | Spraberry, Dean | Iodine ~55–62% Brine ~13–25% | Best overall. Spraberry hits both: strong iodine + CaCl₂-grade brine after extraction. |

## 1.6 National Security Angle — Iodine

- U.S. imports 75% of its iodine. Primary sources: Chile (caliche ore), Japan (brine).

- Underpins medical imaging: MRI contrast agents, CT scans, X-ray contrast media.

- USGS refuses to disclose iodine stockpile levels — classified as national security matter.

- Iodine is listed on the USGS Critical Minerals portal even though not formally designated critical.

- Domestic iodine production, even at pilot scale, addresses a known DOE supply-chain concern.

- Source: USGS Mineral Commodity Summaries 2024, Iodine. USGS National Minerals Information Center.

## 1.7 Extraction Feasibility — Neighbors (Hexagon)

Neighbors/Hexagon technology: nano-dust-coated sand media. Water passes through coated substrate, target ions adsorb to functionalized surface. Backwash recovers concentrated mineral stream.

- Selective for halogens (I⁻, Br⁻) — should NOT strip Ca, preserving kill brine grade.

- Current TRL: ~4–5 (lab-validated, not at field scale). 100 bpd would be a home run at current maturity.

- Competing ions are the main risk: sodium is chemically similar to lithium; sulfate competes with iodide. Real produced water has orders of magnitude more competing ions than lab solutions.

- Proposal framing: pilot-scale slipstream (100–200 bpd) validates extraction at TRL 5 → 7, with techno-economic pathway to full-scale by project year 4–5.

# 2. Kill Brine / Heavy Completion Fluid

## 2.1 What the Box Already Produces

At 70% evaporation, median Permian feed water (87 g/L TDS) concentrates to ~285 g/L TDS, producing 9.9 ppg brine — CaCl₂-grade kill fluid. Distribution across 8,406 Permian wells:

| Brine Grade | ppg Range | % of Permian Wells (at 70% evap) | Application |
| --- | --- | --- | --- |
| Dilute brine | <9.5 ppg | 14.7% | Not viable for completions |
| Light brine | 9.5–10.0 ppg | 22.0% | Limited workover use |
| NaCl kill brine | 9.5–10.0 ppg | 16.6% | Shallow/low-pressure workovers (most common) |
| Low-end CaCl₂ | 10.0–10.8 ppg | 31.1% | Moderate pressure wells |
| CaCl₂ brine | 10.8–11.6 ppg | 12.8% | Medium pressure wells |
| CaCl₂/Ca(NO₃)₂ blend | 11.6–12.4 ppg | 2.3% | Heavy completions |
| NaBr brine | 12.4–13.3 ppg | 0.5% | High pressure |
| CaBr₂ brine | 13.3–14.2 ppg | 0.05% | HPHT wells |

Key result: 62.8% of Permian wells produce commercially viable kill brine (NaCl-grade or better) at 70% evaporation. At 85% evaporation, that rises to ~78%.

## 2.2 Kill Brine = Chemistry, Not Density

PITFALL: Do NOT call brine 'CaCl₂-grade' based on ppg alone. Actual calcium content determines the grade. A 11 ppg brine with low Ca is just heavy NaCl — unsellable as CaCl₂.

| Ca in Feed (mg/L) | Ca in Brine at 10× (g/L) | True Grade | Example Formations |
| --- | --- | --- | --- |
| >5,000 | >50 | Heavy CaCl₂ | Cherry Canyon (14,610), McKee (13,505), Odom (12,217) |
| 2,000–5,000 | 20–50 | CaCl₂-grade | Spraberry (2,854), Wolfcamp (3,690), Atoka (2,100) |
| 800–2,000 | 8–20 | Marginal | Delaware Mtn (1,508 avg), many Wolfcamp wells |
| <800 | <8 | NaCl-only | Cline Shale (673), Cisco (565) |

The ews_formation_catalog.csv labels were assigned by density (ppg), not chemistry. Several are misleading — Cisco (565 mg/L Ca) labeled 'Low-end CaCl₂' is actually NaCl-only. Always cross-check Ca concentration before making grade claims.

## 2.3 Commercial Specifications for Kill Fluid

Industry standards: ISO 13503-3 (Testing of heavy brines), API RP 13J (Testing of heavy brines). Key parameters completion engineers require:

| Parameter | Typical Spec | Standard | What EWS Needs to Verify |
| --- | --- | --- | --- |
| Density | 8.4–19.2 ppg (±0.1 ppg tolerance) | ISO 13503-3 §5 API RP 13J §5 | VOX output density is predictable from feed TDS. Model shows 9.9 ppg median at 70% evap. Need ±0.1 ppg control. |
| Clarity / NTU | <25–50 NTU | API RP 13J §7 | VOX output will contain precipitated barite/celestite/gypsum. Filtration required to meet clarity spec. |
| Crystallization temperature | Must remain liquid at BHT and surface | ISO 13503-3 §8 | CaCl₂ brines crystallize at −20° to −50°C depending on concentration — not a Permian issue. |
| pH | Typically 6–9 (formation-dependent) | API RP 13J §6 | VOX evaporation may drop pH via MgCl₂/CaCl₂ hydrolysis → HCl. May need pH adjustment post-evaporation. |
| Total suspended solids (TSS) | <50–100 mg/L | API RP 13J §7 | Precipitated salts from concentration must be removed. Filtration + settling. |
| Iron content | <10–50 mg/L (formation-dependent) | ISO 13503-3 §9 | Permian feed has median 17 mg/L Fe. Post-evap ~56 mg/L. May need iron removal for sensitive formations. |
| Compatibility with formation | No formation damage (swelling, scaling) | API RP 13J §10 | Site-specific core flood testing. CaCl₂ brines generally compatible with most sandstone/carbonate formations. |

## 2.4 Conditioning Gap — What the Box Output Needs

The VOX box does not produce specification-grade kill fluid directly. The output is hot, concentrated brine with precipitated solids. Conditioning requires:

- 1. Cooling — output is at 80–90°C (operating temp). Needs heat exchanger or holding tank to bring to ambient.

- 2. Solids removal — gypsum (CaSO₄), barite (BaSO₄), celestite (SrSO₄), and possibly halite (NaCl) precipitate during evaporation. Requires filtration (bag/cartridge, possibly diatomaceous earth).

- 3. Density adjustment — if target is specific ppg, may need dilution or further evaporation. CaCl₂ addition if Ca content is below spec.

- 4. pH adjustment — MgCl₂/CaCl₂ hydrolysis during thermal evaporation generates HCl. May need caustic or lime addition to bring pH to 6–9.

- 5. Iron removal — if formation is iron-sensitive, oxidation + filtration or chelation.

- 6. Biocide — if stored before use, produced water brine may have bacterial growth.

- 7. Quality testing — density (mud balance), clarity (turbidimeter), pH, filtration test (API fluid loss). Per ISO 13503-3 / API RP 13J.

These are standard oilfield unit operations — not novel technology. The budget should include a conditioning skid: heat exchanger, settling tank, cartridge filter, chemical injection (pH, biocide), and instrumentation. Estimated cost: $150–400K per pad depending on throughput.

## 2.5 Market Size — Completion Fluid Demand in the Permian

Status: NEEDS VERIFICATION. Ian cited 30–40,000 bpd as the total Permian kill fluid market. This figure requires confirmation.

- TETRA Technologies (NYSE: TTI) is the largest completion fluids provider in the Permian. Their 2023 10-K reports ~$300M+ annual completion fluids & services revenue.

- At typical pricing ($20–80/bbl for CaCl₂ brine, $100–300+ for CaBr₂/ZnBr₂), 30–40K bpd represents a $220M–$350M annual market at the low end.

- Each Permian completion uses 5,000–50,000 bbl of completion fluid depending on well depth and pressure.

- With ~400–500 active frac crews in the Permian (2024), and each completing ~2 wells/month, that's ~1,000 completions/month × 10,000 bbl avg = 10M bbl/month ≈ 330K bpd. This is total completion fluid, not just heavy brine.

- The kill-weight brine segment (10.0+ ppg) is a subset of this — likely in the 50–150K bpd range, not 30–40K.

- Action: confirm with TETRA IR, industry contacts, or SPE market reports. The number matters for the techno-economic narrative.

## 2.6 What We Need (Research Gaps)

These items must be resolved to close the kill brine narrative in the proposal:

| Gap | Priority | Who | Action |
| --- | --- | --- | --- |
| Kill fluid commercial spec confirmation | HIGH | Jesse / Industry contact | Get actual spec sheets from a completion fluids company (TETRA, IW Technologies, or an operator). Verify the ISO 13503-3 / API RP 13J parameters above against real commercial requirements. |
| Market size verification | HIGH | Jesse / Hal | Confirm Permian kill fluid demand. Is it 30–40K bpd or more? TETRA 10-K, SPE papers, or direct call to industry contacts. |
| Conditioning skid cost estimate | MED | Jesse / Russell | Get vendor quotes for heat exchanger + filtration + chemical injection skid for 300–1,000 bpd brine output. This is a budget line item not currently included. |
| Kill fluid consultant / partner | HIGH | Jesse / Hal / Ian | Find a completions engineer or specialty chemical company willing to put their resume/letter in the proposal as kill fluid domain expert. Steep Shiner may have contacts. |
| Ca content of VOX output brine | MED | Jesse / Russell | Existing data is modeled from USGS feed composition. Need actual brine chemistry from a VOX unit to confirm Ca concentration and kill brine grade. |
| Compatibility testing plan | LOW | Proposal narrative | Describe core flood testing with target formation cores. Not needed for submission — described as year 1 activity. |

## 2.7 Economics — Kill Brine Revenue

Brine revenue is the smallest of three revenue streams, but it closes the 'what happens to the water' question for DOE reviewers:

| Revenue Stream | ¢/bbl feed | $/day (10K bpd) | $/year | Notes |
| --- | --- | --- | --- | --- |
| Disposal fee (avoided) | 75–125¢ | $7,500–$12,500 | $2.7–$4.6M | Current EWS rate. DOE proposal = discounted. |
| Iodine (Spraberry median) | 56.3¢ | $5,630 | $2.05M | Dominant mineral revenue |
| Bromine | 32.1¢ | $3,210 | $1.17M | Oilfield fluids — captive demand |
| Kill Brine (CaCl₂) | ~40¢ | ~$4,000 | ~$1.46M | After I+Br extraction @ $1.50–2.50/bbl brine margin |
| TOTAL | ~203–253¢ | ~$20–25K/day | ~$7.4–$9.3M/yr | Per 10K bpd pad, pre-DOE discount |

# 3. Recommended DOE Proposal Framing

## 3.1 Target Formations for Field Sites

| Rank | Formation | Why |
| --- | --- | --- |
| 1 ⭐ | Spraberry (Midland) | Best all-around: 56.3¢/bbl iodine + CaCl₂-grade brine after extraction. 144 g/L TDS — moderate evaporation needed. Existing EWS customers with MSA (Exxon, Continental). |
| 2 | Wolfcamp (Midland) | Higher volume, more wells (430+). 31.3¢/bbl iodine, marginal brine. Good if Spraberry site isn't available. |
| 3 | Atoka (Midland) | 87.1¢/bbl iodine — strong minerals. TDS 69 g/L needs more evaporation. Low-end CaCl₂. Worse brine than Spraberry, better iodine. |
| 4 | Cherry Canyon (Delaware) | Best brine grade (230 g/L TDS, 14,610 Ca). Iodine only 16.6¢/bbl. Good if customer only cares about completion fluid. Delaware generally worse for minerals. |

## 3.2 Extraction Sequence for Proposal

- FEED: 10,000 bpd produced water from Spraberry/Wolfcamp target well

- EVAPORATE: 70% volume reduction via VOX 1000 units (10 units × 1,000 bpd each)

- SLIPSTREAM: 100–200 bpd to Neighbors/Hexagon mineral extraction (I + Br)

- CONDITION: Remaining brine (9,500 bpd × 0.3 = 2,850 bpd brine output) to kill fluid spec via conditioning skid

- VALIDATE: Year 1–2 = 3 units deployed, TRL 5→7. Year 3–5 = full 10 units, continuous operation, techno-economic validation.

## 3.3 Budget Line Items Not Yet Included

- Mineral extraction skid (Neighbors/Hexagon) — slipstream unit + media + backwash recovery: $TBD from partner

- Kill brine conditioning skid — heat exchanger, filtration, pH adjustment, instrumention: $150–400K

- Laboratory analysis — feed water chemistry (monthly), brine output chemistry (monthly), mineral assay: $50–100K/yr

- Kill fluid consultant / domain expert — retainer or in-kind contribution: $TBD

- Core flood / formation compatibility testing — year 1 activity at university partner: ~$100–200K

- Inline sensors — SG/conductivity probes on brine inlet/outlet: $7–13K per unit (10 units = $70–130K)

Energy Water Solutions — CONFIDENTIAL — DOE Grant Proposal Technical Reference

Data: USGS PWDB v2.3 | Mass balance model v2.0 | August 2026